FVT											16.06.2015

Definition of Inland ASM “Forward to Vessel Position Information Server (VPI)”

This message should be sent every 15 minutes (configurable) or when on data field “Forward to VPI” is changed.

|  | Parameter | Bit | Description |
| --- | --- | --- | --- |
|  | Message ID | 6 | Identifier for Message 8; always 8 |
|  | Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Default = 0; 3 = do not repeat any more |
|  | Source ID | 30 | MMSI number |
|  | Spare | 2 | not used, should be set to zero |
| Binary data | Application Identifier | 16 | DAC = 218, FI = 01 (test) |
|  | Version identifier | 3 | Version identifier of ASM = 000 |
|  | ENI / IMO | 1 | 0 = ENI number (default) 1 = IMO number. |
|  | Unique European vessel identification number or IMO number | 30 | ENI: 8 digits number, 1000000 to 99999999, rest not used, if 7 digits a leading zero shall be assumed. 00000000 = ENI not assigned = default IMO: 0 = not available = default – not applicable for SAR aircraft 0000000001-0000999999 not used 0001000000-0009999999 = valid IMO number; 0010000000-1073741823 = official flag state number |
|  | Longitude | 28 | Longitude in 1/10 000 min (±180o, East = positive (as per 2’s complement), West = negative (as per 2’s complement). 181o (6791AC0 hex) = not available = default) |
|  | Latitude | 27 | Latitude in 1/10 000 min (± 90o, North = positive (as per 2’s complement), South = negative (as per 2’s complement), 91o (3412140 hex) = not available = default) |
|  | Data to be shared | 3 | 0 = no information (default) 1 = VTT data only, 2 = ERI data only, 3 = VTT and ERI data 4 to 7 = not used / reserved for future use |
|  | Forward to VPI | 2 | 0 = no information (default) 1 = Yes 2 = No 3 = I don’t care |
|  | spare | 18 | not used. Should be set to zero, reserved for future use |
|  |  | 168 | occupies 1 slot |